“Learning without thought is labor lost; thought without learning is perilous.”

-Confucius, 6th Century B.C.

Chapter 1- Introduction to Design

-1.1 Design

-Objective: ME 314 deals with the application of mathematics, material engineering, mechanics, & mechanics ( or strength) of materials to the design of mechanical components.

-Machine Design deals with the creation of machinery that works safely, reliably, and well.

-It is the engineer’s task to define and calculate those motions, forces, and changes in energy in order to determine the sizes, shapes, and materials needed for each of the interrelated parts in the machine.

-Mechanical Components are machine elements

-Machine is: 	(1) An apparatus with interrelated units, or

(2) A device that modifies force or motion

“The designer cannot predict with accuracy what environmental loads the user will subject the machine to… The dilemma can be resolved only by iteration, which means to repeat, or to return to a previous state.

We must assume some trial configuration for each part, use the mass properties (mass, CG location, and mass moment of inertia) of that trial configuration in a dynamic force analysis to determine the forces, moments, and torques acting on the part, and then use the cross-sectional geometry of the trial design to calculate the resulting stresses.

-1.2 A Design Process

| Table 1-1 | A Design Process |  |
| --- | --- | --- |
| 1 | Identification of Need | Problem statement (usually vague) |
| 2 | Background Research | Needed to fully define the problem |
| 3 | Goal Statement | Restate the goal realistically |
| 4 | Task Specifications | A detailed set of tasks to bound the scope of design |
| 5 | Synthesis | As many alternative designs as possible are sought |
| 6 | Analysis | Possible solutions are analyzed and either accepted, rejected, or modified |
| 7 | Selection | Most promising design is selected |
| 8 | Detailed Design | Drawings made, vendors identified, etc. |
| 9 | Prototyping and testing | Actual fabrication is done as a prototype |
| 10 | Production | Prototyped is mass produced in quantity |

-1.3 Problem Formulation and Calculation

Table 1-2 Problem Formulation and Calculation

|  | Process | Stage |
| --- | --- | --- |
| 1 | Define the problem |  |
| 2 | State the givens | Definition stage |
| 3 | Make appropriate assumptions |  |
| 4 | Preliminary design decisions | Preliminary design stage |
| 5 | Design sketches |  |
| 6 | Mathematical models |  |
| 7 | Analysis of the Design | Detailed design stage |
| 8 | Evaluation |  |
| 9 | Document Results | Documentation Stage |

-1.4 The Engineering Model

-The Engineering Model

-The engineering model that we are describing here is an amorphous thing which may consist of some sketches of the geometric configuration and some equations that describe its behavior. It is a mathematical model that describes the physical behavior of the system.

-Real-life engineering design problems are not of this type:

“Given A, B, and C, find D…”

-They are very unstructured and must be structured by you before they can be solved.

-A useful strategy is to recognize that:

1) You must begin somewhere.

2) Wherever you begin, it will probably not be the “best” place to do so.

3) The magic of iteration will allow you to back up, improve your design, and eventually succeed.

-The Engineering Sketch

-This may be a freehand sketch…

-This sketch often serves the primary purpose of communicating the concept to other engineers and even to yourself

-This sketch should, at a minimum, contain three or more orthographic views, aligned according to proper drafting convention…

1.5 Computer- Aided Design And Engineering

-Up to the student to read

1.6 The Engineering Report

-... Engineers spend a large percentage of their time communicating with others, either orally or in writing.

-Engineers write proposals and technical reports, give presentations, and interact with support personnel.

-In addition to a written description of the design, these reports will usually contain engineering drawings or sketches as described earlier, as well as tables and graphs of data calculated from the engineering model.

1.7 Factors of Safety and Design Codes

-Factor of Safety

-It is typically a ratio of two quantities that have the same units, such as strength/stress, critical load/applied load, load to fail part/expected service overload, maximum cycles/applied cycles, or maximum safe speed/operating speed.

-The safety factor can be thought of as a measure of the designer’s uncertainty in the analytical models, failure theories, and material-property data used, and should be chosen accordingly.

-... The safety factor then becomes a reasonable measure of uncertainity.

-...safety factors for commercial aircraft are in the range of 1.2 to 1.5.

-Read rest of Chapter 1.7

1.8 Statistical Considerations

-Read rest of the chapter

1.9 Units

-U.S. foot-pount-second system (FPS)

-U.S. inch-pound-second system (IPS)

-System Internationale (SI)

-Centimeter, gram, second (CGS)

-Base Units

-Units that can be used to describe a system that are not derived from another unit

-SI system chooses mass, length, and time as base units

-Derived Units

-derived from a mixture of base units

-i.e. force is derived from mass, length, and time F=m*lt2

-alternatively, m=F*t2l

-Mass

-In FPS, pounds seconds squared per foot lb*sec2ft=slugs

gravitational constant, gc=32.17 ftsec2

-In IPS, pounds seconds squared per inch lb*sec2inch=blobs

gravitational constant, gc=386 inchsec2

→ 12 blobs = 1 slug

-Additional, pound mass (lbm)

-...used in fluid dynamics and thermodynamics

-F=m*agc

-In SI, newton seconds squared per meter N*sec2meter=kg

-Mass WRT to gravitational constants

→ 1lbm=1*lbf

→ 1 slug=32.17*lbf

→ 1 blob=386*lbf

→ 1 kg=9.81*N

Conversion Factors on Table 1-5

Some of the biggest problems that can occur in engineering are using the incorrect units, here are some examples of incorrect units:

- Mars Climate Orbiter: http://www.cnn.com/TECH/space/9909/30/mars.metric.02/

-Mars Climate and Others: http://spacemath.gsfc.nasa.gov/weekly/6Page53.pdf

-Gimli Glider: http://en.wikipedia.org/wiki/Gimli_Glider

-Others: http://mentalfloss.com/article/25845/quick-6-six-unit-conversion-disasters