Chapter 3- Load Determination

3.1 Degree of Freedom

Degree of Freedom

→ is the number of coordinates needed to define a systems position in space.

→ In three-dimensional space, a single rigid body has six DOF, three translational (x,y,z) and three rotational (θ,φ, γ).

→ A system’s DOF also defines the number of inputs (motors or other actuators) needed to control its motion.

→ DOF of 1, needs one input

→ DOF of 1+, needs multiple inputs

→ DOF of 0, is a structure that cannot move

→ DOF -1, is a preloaded structure

3.2 Mechanisms

Mechanism

Mechanisms come in many types but are all variants of a linkage, which is made of a collection of links and joints, one of which is grounded, and all are interconnected in a way to provide a controlled output in response to one or more inputs.

Links

Is a rigid body of any shape that has some number of attachment points called nodes (which are often holes) that allow multiple links to be connected by joints.

Joints

Are characterized by their geometry, by the number of degrees of freedom they allow between the links they join, and by whether they are held together (closed) by a force or by their form (geometry).

3.3 Calculating Degree of Freedom (Mobility)

The Kutzbach equation uses just the number of links and joints for the calculation

M=3(L-1)-2*J1-J2 								(3.1)

M = degree of freedom or mobility

L = # of links

J1 = number of 1 DOF joints

J2 = number of 2 DOF joints

This equation does not account for link lengths and so can predict an incorrect DOF when the link geometries have particular values.

This is called a paradox of the Kutzbach equation. Many other paradoxes to this equation exist, all due to unique geometries.

3.4 Common 1-DOF Mechanisms

3.4.1 Four bar Linkage and the Grashof Condition

Its behavior can be predicted by the Grashof criterion:

S+L≤P+Q									(3.2)

S = is the length of the shortest link

L = is the length of the longest link

P & Q = is the length of other two links

→ If the Grashof inequality is false, then no link can make a full revolution and it is said to be a non-Grashof Linkage.

→ If true, then at least one link can revolve fully with respect to the others and it is called a Grashof linkage.

→ If the expression evaluates to equal, then it is a special-case Grashof Linkage. Such a linkage either has to be constrained to avoid these change points, or additional links added to carry it through the change points.

→ The Ground link is always 1

→ the input link is always 2

→ the output link is always 4

→ the coupler between links 2 and 4 is 3

→ A crank can fully revolve, a rocker cannot (they oscillate about an arc).

→ Each inversion has the potential to deliver different types of motion from the same set of links.

Above is a special case of Grashof linkages

Above is a non-Grashof Four bar linkage

Here the wheel motion only requires the links to move through relatively small angles and never through a full revolution.

3.6.2 Six bar Linkage

There are two simple forms of the six-bar linkage, Watt and Stephenson.

The figure below are several examples in which the all contain 4 binary links to which a pair of ternary links has been added.

3.4.3 Four bar Crank-Slider and Slider Crank

If the rocker of a four bar crank rocker linkage is increased in length indefinitely, it effectively becomes infinite in length and the linkage is transformed into a four bar crank-slider.

→ The rocker link becomes a slider block that oscillates in a straight line rather than along an arc.

→ It can also be driven backwards as shown in the figure below, this is also how many Internal Combustion (IC) engines work.

3.4.4 Cam and Follower

A cam and follower is another variant of the four bar linkage in which the crank takes on a contoured shape.

→ There is no “physical” coupler link.

One major advantage of a cam-follower mechanism over a pure linkage is the variety of follower motion functions possible.

→ You can think of the cam-follower mechanism as a four bar linkage or crank-slider in which the crank and coupler are able to change their lengths as the cam rotates.

3.5 Analyzing Linkage Motion

Before determining the forces and torques in a linkage, need to first determine the acceleration. Which is found by determining the position and the velocity of the linkages.

3.5.1 Types of Motion

Complex motion in a 3 DOF plane has two components, Translation and Rotation.

→ Complex motion of a link then involves a translation of a point on the link in x and y and a rotation θ about a point in the link.

→ A good example of this is a crank-slider system, the crank is in pure rotation and the slider is in pure translation. The coupler then has both components, rotation and translation.

3.5.2 Complex Numbers as Vectors

Position, velocity, and acceleration are vector quantities and so we need a notation to represent a vector.

Complex numbers provide some advantage.

→ The axes are labeled real and imaginary. The imaginary axis simply depicts the y-directed cartesian component of the vector with the x-directed component on the real axis.

We can easily convert from polar to cartesian coordinates with Euler’s identity:

e ± jθ=cosθ±j*sinθ								(3.3)

The reason for this is the ease of differentiation.

r*e jθ*dθ=r*j*e jθ

3.6 Analyzing the Four bar Linkage

3.6.1 Solving for position in the four bar linkage

One main thing about using vector notation for analyzing a four bar linkage is that the sum of the vectors equal zero.

The first step is to sum the vectors around the loop, taking their orientations into account.

R2+R3-R4-R1=0									(3.4a)

Substituting vectors R with the letters to represent the link magnitudes.

a*ejθ2+b*ejθ3-c*ejθ4-d*ejθ1=0						(3.4b)

Converting polar to cartesian form

a(cosθ2+jsinθ2)+b(cosθ3+jsinθ3)-c(cosθ4+jsinθ4)-d(cosθ1+jsinθ1)=0	(3.4c)

if θ1=0, x-components:

a*cosθ2+b*cosθ3+c*cosθ4-d=0

if θ1=0, y-components:

a*sinθ2+b*sinθ3+c*sinθ4-d=0

If you input θ2, we can find θ4 by:

θ41,2=2*arctan-B±B2-4*A*C2*A

A=cosθ2-K1-K2*cosθ2+K3

B= -2*sinθ2

C= K1-(K2+1)cosθ2+K3

K1=da,        K2=dc,        K3=a2-b2+c2+d22*a*c

If you input θ2, we can find θ3 by:

θ31,2=2*arctan-E±E2-4*D*F2*D

D=cosθ2-K1-K4*cosθ2+K5

E= -2*sinθ2

F= K1+(K4-1)cosθ2+K5

K1=da,        K4=db,        K5=-a2-b2+c2-d22*a*b

3.6.2 Solving for Velocity in the four bar linkage

R2+R3-R4-R1=0									(3.4a)

Substituting vectors R with the letters to represent the link magnitudes.

a*ejθ2+b*ejθ3-c*ejθ4-d*ejθ1=0						(3.4b)

Differentiate equation 3.4b with respect to time to get velocity.

j*a*ejθ2dθ2dt+j*b*ejθ3dθ3dt-j*c*ejθ4dθ4dt=0					(3.6a)

dθ2dt=ω2 ,    dθ3dt=ω3 ,    dθ4dt=ω4 ,						(3.6b)

j*a*ω2*ejθ2+j*b*ω3*ejθ3-j*c*ω4*ejθ4=0					(3.6c)

Another Form known as the velocity difference equation can be written as:

VA+VBA-VB=0

VA=j*a*ω2ejθ2=a*ω2(-sinθ2+j*cosθ2)

VBA=j*b*ω3ej3=b*ω3(-sinθ3+j*cosθ3)

VB=j*c*ω4ejθ4=c*ω4(-sinθ4+j*cosθ4)

Solving for angular velocities ω3 and ω4 :

ω3=a*ω2bsin(θ4-θ2)sin(θ3-θ4)				ω4=a*ω2csin(θ2-θ3)sin(θ4-θ3)

3.6.3 Angular Velocity Ratio and Mechanical Advantage

Angular velocity ratio mv is defined as the output angular velocity divided by the input angular velocity.

Four bar linkage and gears: 	mv=ω4ω2

Power Equation for rotating systems:	P=T*ω

Torque ratio:	mT=ToutTin=ωinωout

Mechanical Advantage:		mA=FoutFin=ωin*rinωout*rout

3.6.4 Solving for Acceleration in the four bar linkage

*I will posting the rest of the acceleration analysis on backboard, you will need to read it in order to determine the forces as well as the stresses a four bar linkage will see.

3.7 Analyzing the four bar crank-slider

* I will be posting the four bar crank-slider mathematical breakdown on blackboard, you will need to read it in order to solve the crank-slider homeworks.

3.8 Cam Design and Analysis

* I will be posting the cam-slider mathematical breakdown on blackboard, you will need to read it in order to solve the cam-slider homeworks.

However, at this point, we may come back to cam designs later in the future due to time constraints.

3.10 - Free-Body Diagrams

-In order to correctly identify all potential forces and moments on a system, it is necessary to draw accurate free-body diagrams (FBDs) of each member of the system.

-There may be external forces and moments applied to the part from outside the system, and there will be interconnection forces and/or moments where each part joins or contacts adjacent parts in the assembly or system.

→ Remember, the dimensions and angles of the elements in the system are defined with respect to the local coordinate systems located at the center of gravity (CG) of each element.

-For a dynamic load analysis, the kinematics accelerations, both angular and linear (at the CG), need to be known or calculated for each element prior to doing the load analysis.

An inversion results from the grounding of a different link in the chain, thus any linkage will have as many inversions as it has links.

3.11 - Load Analysis

-This section presents a breif review of Newton’s Laws and Euler’s equations as applied to dynamically loaded and statically loaded systems in both 3-D and 2-D.

-This approach assumes all unknown forces and moments on the system to be positive in sign, regardless of what one’s intuition or an inspection of the free-body diagram might indicate as to their probable directions.

-Even with traditional approach, an incorrect assumption of direction results in a sign reversal on that component in the solution.

-- Assuming all unkown forces and moments to be positive allows the resulting computer program to be simpler than would otherwise be the case.

-Real dynamic systems are three dimensional and thus must be analyzed as such.

--However, many 3-D systems can be analyzed by simpler 2-D methods.

3.11.1- Three-Dimensional Analysis

-Dynamic load analysis can be done by any of several methods, but the one that gives the most information about internal forces is the Newtonian approach based on Newton’s laws.

--Newton’s first law- A body at rest tends to remain at rest and a body in motion at constant velocity will tend to maintain that velocity unless acted upon by an external force.

--Newton’s second law- The time rate of change of momentum of a body is equal to the magnitude of the applied force and acts in the direction of the force.

For linear forces:

F=ma

F= force

m= mass

a= acceleration or dVdt or d2xdt2

For moments or torques:

MG=H'G

MG=moment about the center of gravity

H'G= the time rate of change of the moment of momentum, or the

angular momentum about the CG

→ The left sides of these equations respectively sum all the forces and moments that act on the body, whether from known applied force or from interconnections with adjacent bodies in the system.

-If the x,y,z axes are chosen coincident with the principal axes of inertia of the body, the angular moment of a body is defined as:

HG=Ixwxi+Iywyj+Izwzk

where Ix,Iy, and Izare the principle centroid mass moments of inertia (second moments of mass) about the principal axes.

With that said:

Linear forces acting along the x, y, and z axes are:

Fx=max

Fy=may

Fz=maz

Moments about the x, y, and z axes are

Mx=Ixαx-(Iy-Iz)wywz

My=Iyαy-(Iz-Ix)wzwx

Mz=Izαz-(Ix-Iy)wxwy

*It should be noted that these are scalar equations, as known as Euler’s Equations.

Figure - 3 Dimensional coordinate system

--Newton’s Third Law- That when two particles interact, a pair of equal and opposite reaction forces will exist at their contact point. This force pair will have the same magnitude and act along the same direction line, but have opposite sense.

3.11.2 Two-Dimensional Analysis

All real machines exist in three dimensions but many three-dimensional systems can be analyzed two dimensionally if their motions exist only in one plane or in parallel planes.

For instance, if we are only concerned about the XY plane, the 6 governing equations reduces to three governing equations

Fx=max		Fy=may		Mz=Izαz

The remaining three equations are therefore equal to zero:

Fz=0		Mx=0		My=0

Figure - Two Dimensional Coordinate system

3.11.3- Static Load Analysis

The difference between a dynamic loading situation and a static one is the presence or absence of accelerations.

--For a three-dimensional case, all the equations reduce to zero:

Fx=0	Fy=0	Fz=0	, all linear forces equal zero

Mx=0	My=0	Mz=0	, all moments equal zero

Please Read Chapters 3.12, 3.13, and 3.14. They are case studies how to analyze static and dynamic systems.

3.15 Vibration Loading

In systems that are dynamically loaded, there will usually be vibration loads superimposed on the theoretical  loads predicted by the dynamic equations.

→ All real elements, of any material, have elasticity and thus act as springs when subjected to forces.

→ The resulting deflections can cause additional forces to be generated from the inertial forces associated with the vibratory movements of elements or, if clearances allow contact of mating parts, to generate impact (shock) loads during their vibrations.

If not using FEA, we would like to determine, at a minimum, the system’s lowest, or fundamental, natural frequency, since this frequency will usually create the largest magnitude of vibrations.

The undamped fundamental natural frequency ωn, with units of rad/sec, of fn, with units of Hz.

ωn=km		fn=12πωn

k is the spring constant, k=Fδ=Fy

If there is damping (frictional losses with respect to velocity), then the frequency will change to a damped natural frequency.

ωd=km-d2*m2

fd=12πωd

d=Fy '

Resonance

Resonance is the condition when the input forcing frequency is the same as the natural frequency of the system.

→ This can create large forces and cause failure.

3.16 Impact Loading

-Many machines have elements that are subjected to sudden loads or impacts.

-Severe Collisions between moving objects can result in permanent deformation of the colliding bodies as in an automobile accident. In such cases the permanent deformation is desirable in order to absorb the large amount of energy of the collision and protect the occupants from more severe harm.

-Two general cases of impact loading are considered to exist...these two cases [are called] striking impact and force impact.

-Striking Impact refers to an actual collision of two bodies, such as in hammering or the taking up of clearance between mating parts.

i.e. pressure pushing up on piston head and/or piston head in contact with the cylinder walls in a combustion engine.

-Force Impact refers to a suddenly applied load with no velocity of collision, as in a weight suddenly being taken up by a support.

-i.e. friction clutches and brakes

-...the kinetic energy possessed by the striking object can be equated to the energy stored elastically in the struck object at its maximum deflection. This energy approach gives an approximate value for the impact loading.

--It is not exact because it assumes that the stresses throughout the impacted member reach peak values at the same time. However, waves of stress are set up in the struck body, travel through it at the speed of sound, and reflect from the boundaries.

3.16.1- Energy Method

-The kinetic energy of the striking body will be converted to stored potential energy in the struck body, assuming that no energy is lost to heat.

--If we assume that all particles of the combined bodies come to rest at the same instant, then just before rebound, the force, stress, and deflection in the struck body will be maximal.

E=12Fiδi=Fi22k

-Please read the rest of section 3.16.1, it is very important however due to time constraints we can’t cover it too much in depth.

3.17- Beam Loading

-Beams are typically analyzed as static devices, through vibrations, and accelerations can cause dynamically loadings.

-There are several classes of beams, but we will focus on four types:

-Simply Supported, a beam supported on pins or narrow supports at each end

-Cantilever Beam- A beam fixed at one end and unsupported on the other

-Overhung Beam- A simply supported beam that overhangs its support at either end.

-Indeterminate Beam- Cannot be solved for its loads using only the three equations.

3.17.1 Shear and Moment

-When applying a force (whether concentrated or distributed) to a beam, the applied forces will create both shearing forces and bending moments in the beam.

-A load analysis must find the magnitudes and spatial distributions of these shear forces and bending moments on the beam.

q(x)=dVdx=d2Mdx2	q(x) is loading function, V is shear, and M is moment

-The loading function q(x) is typically known and the shear V distributions can be found by integrating equation:

VAVBdV=xAxBqdx=VB-VA

-If we can find the moment distributions by integrating the Shear equation above

MAMBdM=xAxBVdx=MB-MA

-Sign Convention

-The usual (and arbitrary) sign convention used for beams is to consider a moment positive if it causes the beam to deflect concave downward.

3.17.2 Singularity Functions

-Because the loads on beams typically consist of collections of discrete entities, such as point loads or segments of distributed loads that can be discontinuous over the beam length, it is difficult to represent these discrete functions with equations that are valid over the entire continuum of beam length.

→ ...singularity functions was invented to deal with these mathematical situations (discontinuous functions).

-The first quantity in the brackets is the variable of interest, in our case x, the distance along the beam length.

-The second quantity a is a user-defined parameter that denotes where in x the singularity function either acts or begins to act.

-Unit Parabolic functions, x-a2

-Used for quadratically distributed loads.

-Defined as 0 when x≤a, and equal to (x-a)2 when x>a.

-Unit ramp function, x-a^1

-Used for linearly distributed loads.

-Defined as 0 when x≤a, and equal to (x-a) when x>a

-Unit step function, x-a0

-A uniformly distributed load over a portion of a beam

-Defined as 0 when x≤a, unity when x>aand is undefined at x=a

-Unit impulse function, x-a-1

-A concentrated force, or point load.

-Defined as 0 when x<a, ∞ when x=a, and 0 when x>a

-Its integral evaluates to unity at a

-Unit doublet function, x-a-2

-A concentrated moment, or point-moment.

-Defined as 0 when x<a, indeterminate when x=a, and 0 when x>a

-It generates a couple moment at a

-The integral of these singularity functions have special definitions that, in some cases defy common sense but nevertheless provide the desired mathematical results.

-∞x{λ-a}2dλ={x-a}33

-∞x{λ-a}1dλ={x-a}22

-∞x{λ-a}0dλ={x-a}1

-∞x{λ-a}-1dλ={x-a}0

-∞x{λ-a}-2dλ={x-a}-1