Chapter 4.0 Stress, Strain, and Deflection

4.1- Stress

-Stress was defined in Chapter 2 as force per unit area with units of psi or MPa.

-In a part subjected to some forces, stress is generally distributed as a continuously varying function within the continuum of material.

-Every infinitesimal element of the material can conceivably experience different stresses at the same time.

-These infinitesimal elements are typically modeled as cubes.

Normal Stresses(σ)- act perpendicular (i.e. normal) to the face of the cube and tend to either pull it out (tensile normal stress) or push it in (compressive normal stress).

Shear Stresses (τ)- act parallel to the faces of the cubes, in pairs (couples) on opposite faces, which tends to distort the cube into a rhomboidal shape.

-These normal and shear components of stress acting on an infinitesimal element make up the terms of a tensor.

-Stress is a tensor of order two and thus requires nine values or components to describe it in three dimensions.

-3-D stress tensor

| σxx | τxy | τxz |
| --- | --- | --- |
| τyx | σyy | τyz |
| τzx | τzy | σzz |

-The first subscript designates the reference face, and the second subscript designates the direction of the stress.

-2-D Stress Tensor

| σxx | τxy |
| --- | --- |
| τyx | σyy |

-It should be noted that the stress tensor is symmetric, which means that:

τxy=τyx	τyz=τzy	τzx=τxz

4.2 Strain

Stress and strain are linearly related by Hooke’s law in the elastic region of most engineering materials as discussed in chapter 2.

ε=σE

-Strain is also a second-order tensor and can be expressed for the 3-D case:

| εxx | εxy | εxz |
| --- | --- | --- |
| εyx | εyy | εyz |
| εzx | εzy | εzz |

-Strain for a 2-D case:

| εxx | εxy |
| --- | --- |
| εyx | εyy |

-The same symmetric relationships shown for shear stress components also apply to the strain components.

4.3 Principal Stresses

-Since, from an engineering standpoint, we are most concerned with designing our machine parts so that they will not fail, and since failure will occur if the stress at any point exceeds some safe value, we need to find the largest stresses (both normal and shear) that occur anywhere in the continuum of material that makes up our machine part.

-There will always be planes on which the shear-stress components are zero.

-The normal stresses acting on these planes are called the principal stresses.

-The planes on which these principal stresses act are called the principal planes.

-The directions of the surface normals to the principal planes are called the principal axes, and the normal stresses acting in those directions are the principal normal stresses.

-There will also be another set of mutually perpendicular axes along with the shear stresses will be maximal.

-The principal shear stresses act on a set of planes that are 45oangles to the planes of the principal normal stresses.

-The expression relating the applied stresses to the principal stresses is

| σx-σ | τxy | τxz | nx |  | 0 |
| --- | --- | --- | --- | --- | --- |
| τyx | σy-σ | τyz | ny | = | 0 |
| τzx | τzy | σz-σ | nz |  | 0 |

n*n=1	n=nxi+nyj+nzk

-Expanding this determinant and setting it to zero, we obtain:	*3-D cases*

σ3-C2σ2-C1σ-C0=0

Where:	C2=σx+σy+σz

C1=τxy2+τyz2+τzx2-σxσy-σyσz-σzσx

C0=σxσyσz+2τxyτyzτzx-σxτyz2-σyτzx2-σzτxy2

-The coefficients C0, C1, and C2are called the tensor invariants, because they have the same values regardless of the initial choice of xyz axes in which the applied stresses were measured or calculated.

-The three principal (normal) stresses σ1, σ2, and σ3are the three roots of this cubic polynomial.

-The roots of this polynomial are always real and are usually ordered such that σ1>σ2>σ3.

-The directions of the three principal stresses are mutually orthogonal.

-The principal shear stresses can be found from the values of the principal normal stresses using:

τ13=σ1-σ32

τ21=σ2-σ12

τ32=σ3-σ22

-The directions of the planes of the principal shear stresses are 45ofrom those of the principal normal stresses and are also mutually orthogonal.

-For the special case of a two-dimensional stress state for principal stress reduce to:	*Two-Dimensional Cases, muy importante!!!*

σa, σb=σx+σy2±σx-σy22+τxy2		σc=0

σa=σ1=algebraically largest

σb=σ3=algebraically smallest

σc=σ2=0 (other)

-Once the three principal stresses are found and ordered as described above, the maximum shear stress is:

τmax=τ13=σ1-σ32

-... the 3-D case can still be used to solve any two-dimensional case.

4.4 Plane Stress and Plane Strain

4.4.1 Plane Stress

Plane Stress	- two dimensional, or biaxial, stress state.

-Requires that one principal stress be zero.

4.4.2 Plane Strain

Plane Strain	-There are principal strains associated with the principal stresses.

-The cross-sectional strains are far greater than the strains along the “z” direction (or along the length of the material), i.e. negligible.

4.5 Mohr’s Circle

-Mohr’s circles have long provided a means to do a graphical solution and find the principal stresses for the plane stress case.

-Generally for two dimensional cases.

-Mohr’s circles exist for the three-dimensional stress case as well, but a graphical construction method is not available to create them directly from the applied-stress data except for the special case where one of the principal stresses is coincident with an axis of the xyz coordinate system chosen, i.e., where one plane is a plane of principal stress.

-The Mohr plane, on which Mohr’s circles are drawn, is arranged with its axes drawn mutually perpendicular, but the angle between them represents 180oin real space.

-All angles drawn on the Mohr plane are double their value in real space.

-The abscissa (mathematical name for horizontal axis) is the axis of all normal stresses. The applied normal stresses σx, σy, and σzare plotted along this axis and the principal stresses σ1, σ2, and σ3 are also found on this axis.

-The ordinate (mathematical name for the vertical axis) is the axis of all shear stresses. It is used to plot the applied shear stresses τxy, τyx, and τxz and to find the maximum shear stress.

-In layman’s terms, remember that all σ'sare plotted on the horizontal axis whether they are applied normal stresses (σx, σy, σz) or principal stresses (σ1, σ2, σ3) and all τ's are plotted on the vertical axis whether they are applied shear stresses (τxy, etc.) or maximum shear stresses (τ12, etc.).

-Easier way of drawing Mohr’s circle is determining the center of Mohr’s circle and the radius:

C=σx+σy2         {center}

R=σx-σy22+τxy2                        {radius}

σa=σ1=C+R,	σb=σ3=C-R,	σc=σ2=0

*Remember it is Two-Dimensional only, use other methods for 3-D*

4.6 Applied Versus Principal Stresses

Applied Stresses- are the nine components of the stress tensor that result from whatever loads are applied to the particular geometry of the object as defined in a coordinate system chosen for convenience.

Principal Stresses- are the three principal normal stresses and the three principal shear stresses defined section 4.3.

-However, the principal stresses are both normal and shear.

-In the case of an applied stress that is pure tensile, the maximum principal normal stress is equal to applied stress in both direction and magnitude.

-But a principal shear stress of half the magnitude of the applied tensile stress acts on a plane 45ofrom the plane of the principal normal stress.

-Thus, the principal shear stresses will typically be nonzero even in the absence of any applied shear stress. This fact is important to an understanding of why parts fail, and it will be discussed in more detail in Chapter 5.

4.7 Axial Tension

-Axial loading in tension is one of the simplest types of loading that can be applied to an element.

-It is assumed that the load is applied through the area centroid of the element and that the two opposing forces are collinear along the x axis.

-At some distance away from the ends where the forces are applied, the stress distribution across the cross section of the element is essentially uniform.

-Applied normal stress for pure axial tension (or compression)

σx=PA

-Deflection for a purely axial tension (or compression)

Δs=PlAE	→ Δsl=ε=PA1E=σE

-Factor of Safety

N=Syσ1

4.8 Direct Shear Stress, Bearing, and Tearout

These types of loading occur mainly in pin-jointed, bolted, or riveted connections.

-Possible modes of failure are direct shear of the connector (pin, rivet, or bolt), bearing failure of connector or surrounding material, or a tearing out of the material surrounding the connector.

4.8.1 Direct Shear

Direct shear occurs in situations where there is no bending present.

τxy=PAshear

-where P is the applied load and Ashearis the shear-area being cut, i.e., the cross-sectional area being sheared.

-The assumption here is that the shear stress is uniformly distributed over the cross section. This is not accurate, since higher local stresses occur at the blade.

-The Shear blade is tight against the jaws that hold the workpiece. Thus, the two forces P are in the same plane and do not create a couple.

-Shows the same workpiece with a small gap (x) between the shear blade and the jaws. This creates a moment arm, turning the pair of forces P into a couple and thus bending, rather than directly shearing the part.

-There are two general shear types: Single and Double.

-single shear, because only one cross-sectional area of the part needs to be severed to break it.

-double shear, two areas must fail before it separates.

-This is also called a clevis-pin joint

-Double shear is preferred over single shear for pivot-pin designs. Single-shear pivots should only be used where it is impossible to support both ends of the pin as in some linkage cranks.

-Bolted and riveted joints are in single shear when only two flat pieces are fastened together.

4.8.2 Direct Bearing

-A pivot pin in a hole may fail in other ways than in direct shear. The surfaces of the pin and hole are subjected to a direct bearing stress, which is compressive in nature. Bearing stress occurs whenever two surfaces are pressed together.

-This stress tends to crush the hole or pin rather than to shear it.

-The bearing stress is normal, compressive, and can be calculated:

σx=PA

-hole with essentially no clearance, the area used for this calculation is typically taken as the projected area of contact of pin and hole, not the circumferential area.

Abearing=ld

l= length of the bearing contact

d= diameter of the hole or pin

-If there is clearance between the pin and hole then the area of contact is reduced.

Abearing=π4ld

l= link thickness

d= diameter of the pin

4.8.3 Tear-out Failure

-Another possible mode of failure for pinned joints is tear-out of the material surrounding the hole.

-This will occur if the hole is placed too close to the edge.

-This is a double shear failure, as it requires both sides of the hole to separate from the parent material.

4.9 Beams and Bending Stresses

-Any intermittently supported member subjected to loads transverse to its length will act as a beam.

-Beams will usually have some combination of normal and shear stresses distributed over their cross sections.

4.9.1 Beams in Pure Bending

-Most real beams will be subjected to a combination of bending and shear loading

-However, it is first crucial to understand the theory of stresses due to bending loads.

4.9.1.1 Straight Beams

Assumptions for analyzing Beams in pure bending:

1. The segment analyzed is distant from applied loads or external constraints on the beam.

2.The beam is loaded in a plane of symmetry.

3. Cross sections of the beam remain plane and perpendicular to the neutral axis during bending.

4. The material of the beam is homogeneous and obeys Hooke’s law.

5. Stresses remain below the elastic limit and deflections are small.

6. The segment is subjected to pure bending with no axial or shear loads.

7. The beam is initially straight.

-The unloaded segment is straight but, as the bending moment is applied, the segment becomes curver

-The line from N to N along the neutral axis does not change length, but all other lines along the x direction shorten or lengthen in order to keep all cross sections perpendicular to the neutral axis.

-The outer fibers of the beam at the top are shortened, which puts them in compression, and the outer fibers at the bottom are lengthened and put in tension.

-The bending stress magnitude is zero at the neutral axis and is linearly proportional to the distance y from the neutral axis.

σbending= -M*yI

M = the applied moment

y = is the distance from the neutral axis

I = is the second moment of inertia  (area moment of inertia) of the beam cross sectional area

-The maximum bending stress occurs at the outer fibers (as seen in figure below):

σmaximum= M*cI

M = the applied moment

c = is the maximum distance from the neutral axis

I = is the second moment of inertia  (area moment of inertia) of the beam cross sectional area

-An Alternate form is

σmax=MZ

Z= the beam’s section modulus, Z=Ic

-These equations are applicable to cases, where shear strain is negligible, in which other loads in addition to the moment are applied to the beam

4.9.1.2 Curved Beams

-Believe it or not, curved beams are not straight.

-The have a radius of curvature.

-The first six assumptions listed for straight beams still apply.

-If a beam has significant curvature, then the neutral axis will no longer be coincident with the centroid axis and the previous equations do not apply.

-The neutral axis therefore shifts towards the center of curvature by a distance e

e=rc-AdAr

-rc, is the radius of curvature of the curved beam

- A, is the cross-sectional area

- r, is the radius from the beam’s center of curvature to the differential area, da.

-Integration is necessary for complex shapes.

-The stress distribution across the section is no longer linear but is now hyperbolic, and it is greatest on the inner surface of a rectangular cross section.

-Since the convention causes a positive moment that straightens the beam. The causes a tensile stress on the inner surface and a compressive stress on the outer surface.

-For pure bending for this cross-sectional/ shape of beam, the maximum stresses at inner and outer surfaces are:

σi= +Me*Aciri

σo= -Me*Acoro

-if the ratio

4.9.2 Shear Due to Transverse Loading

The more common condition in beam loading is a combination of both shear force and bending moment applied to a particular section

This element is dx wide and is cut in from the outer fiber at c to a depth y1from the neutral axis. This element is dx wide and is cut in from the outer fiber at c to a depth y1from the neutral axis.

-Note that the magnitude of the moment M(x1) on the left-hand side of P(face b1-c1) is less than the moment M(x2) on the right-hand side (face b2-c2), and their difference is the differential moment dM.

...The moment M(x) is increasing as a function of beam length x, due to the presence of the nonzero shear force V at that point.

-Since the normal stress due to bending is proportional to M(x), the stress σ on the left-hand face of P is less than on its right-hand face.

The force acting on the left-hand face of P at any distance y from the neutral axis can be found by multiplying the stress by the differential area dA at that point.

σdA=M*yI*dA

The total force acting on the left-hand face is found by integrating:

F1x=y1cM*yI*dA

The total force acting on the right-hand face:

F2x=y1c(M+dM)*yIdA

Shear force on the top face at distance y1 from the neutral axis:

Fxy=τ*b*dx

b= the width of the beam

For equilibrium, the forces acting on P must sum to zero,

Fxy=F2x-F1x

→ τ*b*dx=y1c(M+dM)*yI*dA-y1cM*yI*dA

→ τ=dMdx*1Iby1cy*dA=VIby1cy*dA=V*QIb

Q=y1cy*dA

4.9.2.1 For Rectangular Beams:

Q=y1cy*dA=by1cy*dy=b2*h24-y12

and τ=V2*I*h24-y12

τmax=32*VA

4.9.2.2 For Round Beams

Q=y1cy*dA=2y1cy*r2-y2*dy=23*r2-y1232

Shear Stress Distribution

τ=V*Qb*I=V23r2-y12322r2-y2π*r44=43*Vπ*r21-y12r2

Maximum shear stress for solid, circular cross-sectional beam

τmax=43*VA

Maximum shear stress for hollow and thin walled

τmax≃2*VA

→ for wall thickness < about 1/10 the outside radius

4.10 Deflection In Beams

Distributed Loads	qE*I=d4ydx4

Shear Loads		VE*I=d 3ydx3

Moment 		ME*I=d 2ydx2

Slope			θ=dydx

Deflection		y=f(x)

4.10.1 Integral forms of the beam equations:

For Shear:		V=q*dx +C1				0<x<l

For Moment:		M=V*dx+C1x+C2			0<x<l

For Slope:		θ=ME*I*dx+C1x2+C2x+C3		0<x<l

For Deflection:	y=θ*dx+C1x3+C2x2+C3x+C4		0<x<l

4.11 CASTIGLIANO’S METHOD

Area- Moment Method - treats the moment function as if it were a “loading”

funcion and integrates twice to obtain the deflection function.

Castigliano’s Method- uses strain energy equations to determine the deflection at any point.

-Energy methods often provide simple and rapid solutions to problems.*

-It can also provide a solution to indeterminate beam problems.

-When an elastic member is deflected by the application of a force, torque or moment, strain energy is stored in the member.

→ When the deflections are small, it can be approximated linearly with the equation below:

U=F*δ2

U= Stored strain energy in the part.

F= is the applied load.

δ= the deflection due to the applied load.

→ However, we need to define deflection as a function strain energy (dependent variable) and a generalized load Q. Castigliano observed that… the deflection in the direct of that load is equal to the partial derivative of the strain energy with respect to that load.

Δ=dUdQ

→ Q is dependent on application*

4.11.0.1 Strain Energy in Axial Loading

For an axial loaded part we know already know the equation for Δ:

Δ=P*lA*E	*From section 4.7

However, if you need the differential form:

Δ=PA*E*dx

Therefore the strain energy for an axial loaded:

Easy form:	U=12*F2lE*A	Differential Form: U=12*0lF2E*A*dx

4.11.0.2 Strain Energy In Torsional Loading

Torsional Strain Energy (more in depth in section 4.12):

U=12*0lT2G*K*dx

T= is the applied Torque

G= is the modulus of rigidity

K= is a geometric property of the cross section

4.11.0.3 Strain Energy In Bending Loading

Strain Energy in Bending:

U=12*0lM2E*I*dx

M = is a bending moment, which may be a function of x

4.11.0.4 Strain Energy In Transverse Shear Loading

-...The strain energy will be a function of the cross-sectional area as well as the load and length.

-For a rectangular cross-sectional beam:

U=35*0lV2G*A*dx

V= is the shear loading, which may be a function of x

- If the beam is non-rectangular section, ½ is often used instead of ⅗ .

*The effects of transverse shear loads on deflections in beams typically will be less than 1% of the effects due to bending moments when the beam’s length-to-depth ratio is >10.

→ Very useful for “short” beams

*if lbeamhbeam>10, 	forget about it! Strain Energy due Shear is negligible.

4.11.1 Deflection by Castigliano’s

-This method is useful for calculating deflections at particular points on a system.

-Where as singularity functions provides the deflection over an entire beam. Which is useful for finding the maximum deflection.

-For a system deflected by more than one type of load, the individual effects can be superposed using a combination of Axial, Torsional, Bending and Shear.

4.11.2 Finding Redundant Reactions with Castigliano’s Method

-Castigliano’s method can be used for solving statically indeterminate problems. by using the Δ equation and setting the supports equal to zero, and solving for the force.

4.12 Torsion

When members are loaded with a moment about a longitudinal axis, they are said to be in torsion, and the applied moment is then called a Torque.

-...The Applied moment vector is parallel to the long axis (neutral axis) of a part rather than transverse (perpendicular) to it as in the case of bending, as seen in the diagram below.

4.12.1 Pure Torsional Loading

*Many machine parts are loaded with combinations of torques and bending moments, which will be dealt with later.

-...a straight bar having uniform circular cross section with a pure torque applied in such manner that no bending moment or other forces are present.

-One end is fixed in a rigid wall or support.

-The other end is free and deflects through an angle θ

Assumptions for Pure Torisonal Loading:

1. The element analyzed is distant from applied loads or external constraints on the bar.

2. The bar is subjected to pure torsion in a plane normal to its axis, and no axial, bending, or direct shear loads are present.

3. Cross sections of the bar remain plane and perpendicular to the axis.

4. The material of the bar is homogeneous, isotropic, and obeys Hooke’s law.

5. Stresses remain below the elastic limit.

6. The bar is initially straight.

4.12.2 Circular Sections

A differential element taken anywhere on the outer surface will be sheared as a result of the torque loading.

The stress τ is pure shear and varies from zero at the center to a maximum at the outer radius.

τ=T*ρJ

T= Applied Torque

ρ= Radius to any point

J= Polar area moment of inertia of the cross section

Therefore the maximum stress will be at the outermost surface, at radius r:

τmax=T*rJ

The angular deflection due to the applied torque is:

θ=T*lJ*G

l=is the length of the bar

G= is the shear modulus (modulus of rigidity) of the material

G=E2(1+ν)

ν=poisson's ratio

Polar Area Moment of Inertia of a Solid Circular cross section

→ J=π*d432

d=diameter of cross sectional area

Polar Area Moment of Inertia of a Hollow Circular cross Section

→ J=π*(do4-di4)32

do=outer diameter of cross sectional area

di=inner diameter of cross sectional area

*The circular cross section is the optimum shape for any bar subjected to torsional loading and should be used in all torsion situations if possible.

4.12.3 Non-Circular Sections

Non-circular cross sections subjected to torsion exhibit behavior that violates some of the assumptions previously listed.

→ Sections do not remain plane, and will warp.

→ Radial lines do not remain straight, and the shear-stress distribution is not necessarily linear across the section.

General Expression for Maximum shear stress due to torsion in non-circular sections:

τmax=TQ

Q= is a function of the cross-sectional geometry

θ=T*lK*G

K= is a function of the cross-sectional geometry

*You may need to check the book for the table that lists expressions for K & Q for some common cross sectional shapes in torsion.

**Any open section is generally no better in torsion than a flat plate of the same cross-sectional dimensions.

***Open Sections should be avoided for all torsionally loaded applications

****Only closed-circular sections, either hollow or solid, are recommended for torsional loading applications.

4.13 Combined Stresses

“It is very common in machine parts to have combinations of loadings that create both normal and shear stresses on the same part.”

-Additionally, “There may be locations within the part where these applied stresses must be combined to find the principal stresses and maximum shear stress.

*Example Problem in class*

4.14 Spring Rates

Rectilinear

krectilinear=Fy=A*El

krectilinear= Force per unit deflection	either N/m or lbf/ft

yrectilinear=F*lA*E

Torsional

ktorsional=Tθ=G*Jl

ktorsional=Torque per unit angular deflection	either in-lbf/rev or N-m/rev

ytorsional=T*lG*J

There will not be a whole lot of time devoted to this but it is a very simple concept and will be left up to the student to read this chapter (1 ½ pages).

4.15 Stress Concentrations

Most real Machine parts have varying cross-sectional areas. Any one of these changes in cross-sectional geometry will cause localized stress concentrations.

-The stress concentrations are highly nonlinear!

-Such geometric changes (fillets, notches, change in diameters, etc) in a part are often called “stress-raisers” and should be avoided or at least minimized as much as possible in design.

-Unfortunately, it is not practical to eliminate all such stress-raisers, since such geometric details are needed to connect mating parts and provide

functional part shapes.

Introducing the new increased maximum stress!!!

σmax=kt*σnom

σnom= Nominal Normal stress either due to axial or bending

kt = denote stress concentration factor for normal stresses

τmax=kts*τnom

τnom= Nomial Shear stress either due to shear loading, shear due to bending, or torsion

kts = denote stress concentration factor for shear stresses

*It should be noted that the factors kt & ktsare only with respect to the part geometry and not how the material behaves in the face of stress concentrations.

→ However, we will be covering how the part behaves in the face of stress concentrations at a later time.

4.15.1 Stress Concentration Under Static Loading

Ductile Materials

will yield locally at the stress-raiser while the lower-stressed material further from the geometric discontinuity remains below the yield point.

→ When the material yields locally, its stress-strain curve there becomes nonlinear and of low slope, which prevents further significant increase in stress at that point.

Brittle Material

will not yield locally, since they do not have a plastic range.

→ Once the stress at the stress-raiser exceeds the fracture strength, a crack beings to form. This reduces the material available to resist the load and also increases the stress concentration in the narrow crack. The part then goes quickly to failure.

→ This is why many bullet resistant armor is effect since it diverts the energy of a bullet into fracturing the ceramic plating, instead of yielding and letting the bullet continue through the armor.

4.15.2 Stress Concentration Under Dynamic Loading

There are two types of Dynamic loading: Impact and Fatigue. Well there is a third on called Creep. But that one may not be covered during this course; but for those who are curious it is dependent on time, atmosphere, and temperature.

While all materials are affected by stress concentrations under dynamic loads, some materials are more sensitive than others.

→ A parameter called notch sensitivity q is defined for various materials and used to modify the geometric factors kt & kts.

→ This subject will be covered when we start to cover Fatigue and Dynamic Failure Analysis.

4.15.3 Determining Geometric Stress-Concentration Factors

The theory of elasticity can be used to derive stress concentration functions for some simple geometries.

→ Review the example on that page.

4.15.3.1 Stress Concentration Measurement

There are more techniques that are being developed via stress/ strain transducers, strain gages, photoelastic techniques, laser holography, and/or other means. However the experimental data have been collected and confirmed, then applied to computer programs such as Finite-Element-Analysis (FEA) and Boundary-Element-Analysis (BEA) to help with Computer Aided Designs (CAD).

4.15.3.2 Effect of Length-Width Ratio

→ * SUPER IMPORTANT. Stress Concentrations are super dependent on the length of the material to the width of features. Or other similar constraints.

4.15.3.3 Stress Concentration Data

→ Talks about how the stress-concentration charts and how to use the graph that will let you know what the stress-concentration factor.

→ Super easy and will be discussed in class on how to use those charts.

→ Most follow the form below:

Kt=A*x b

x= is a variable determined from a geometrical component

A=is a coefficient determined from a given chart

b=is a coefficient determined from a given chart

4.15.3.4 Designing to Avoid Stress Concentrations

The Designer is always faced with the problem of stress concentrations at sections having abrupt changes of a given shape.

→ The best that can be done is to minimize the stress-concentrations effect.

Guidelines for minimizing stress-concentrations:

1. Avoid abrupt and/or large-magnitude changes in cross section if possible

2. Avoid sharp corners completely and provide the largest possible transition radii between surfaces of different contours.

4.15.3.4.1 Force Flow Analogy

“Force Flow” analogy, which considers the forces (and thus the stresses) to flow around contours in a way similar to the flow of an ideal incompressible fluid inside a pipe or duct of changing contour.

→ A sudden narrowing of the pipe or duct causes an increase in fluid velocity at the neckdown to maintain constant flow.

→ Streamlined shapes are used in pipes and ducts to reduce turbulence and resistance to flow.

→ “Streamlining” our part contours can have similar beneficial effects in reducing stress concentrations.

4.16 Axial Compression- Columns

Review:

Axial Stress	σx=PA

When the axial load direction is reversed so as to put the member in compression, the equation alone may not be sufficient to determine the safe load for the member.

Deflection	Δ=P*lA*E

When an axial member is loaded in compression, it is then known as a column.and may fail by buckling rather than by compression.

Buckling occurs suddenly and without warning. The part will resist the load until at some point it suddenly buckles into a bowed shape and collapses.

4.16.1 Slenderness Ratio

Slenderness Ratio (Sr) is a factor that determines if a column is short or long and the equation(s) are shown below:

Sr=lk

l=is the length of the column

k=is the radius of gyration

k=IA

I =the Area moment of Inertia (second moment of area)

A= cross sectional area

4.16.2 Short Column

-A short column will fail in compression and its compressive stress can be calculated from the Axial stress equation from above.

If the Sr<10, then the column is defined as a short column

4.16.3 Long Columns

-An Intermediate or Long column will fail by buckling when the applied axial load exceeds some critical value.

→ The compressive stress can be well below the material’s yield strength at the time of buckling.

-A long column requires the calculation of its critical load (Pcr).

-When the Column starts to bow (ever so slightly), the centroid is then shifted away from the neutral axis, causing a bending moment. Which the bending moment tends to increase the lateral direction of the part, which then increases the distance, which inturns creates an even bigger moment arm (known as positive feedback, important term once you get to ME 330).

-This then causes the part to have both bending and axial stresses.

The Bending moment is given by:

M=P*y

For small deflections of a beam (using equation for section 4.10):

ME*I=d 2ydx2

Combining the two equations above we get a familiar differential equation:

d 2ydx 2+PE*I*y=0

Which gives us a well-known solution:

y=C1*sin(PE*I*x)+C2*cos(PE*I*x)

C1 and C2 are constants of integration that depend on boundary conditions

If we analyze the Solution above with the given boundary conditions

→ y=0 at x=0; y=0 at x=L

→ First, y=0 at x=0:

0=C1*sin(0)+C2*cos(0)=0+C2*1	→ Therefore C2=0

→ Second, y=0 at x=L

0=C1*sinPE*I*L+0   → since C1=0 is trivial, therefore sinPE*I*L=0

→ which will be true for PE*I*L=n*π;  n=1,2,3,...

→ Since failure will occur when n=1, gives us Pcr=π 2*E*IL2

→ Known as Euler-Column Formula for rounded-end or pinned end columns

*Note that the critical load is a function only of the geometry and not the material.

→ The strength of the material is not a factor.

-An alternate form of the critical load, by substituting I=A*k 2

Pcr=π 2*E*A*k 2L 2=π 2*E*ALk2=π 2*E*Asr2

-An Alternate form of the Solution using the first Pcrvalue and the original solution of the differential equation gives us:

y=C1sin(π*xL)

*It should be noted that applying a different end condition and/or different boundary condition will yield a different deflection curve and different critical load. Only works for pinned or rounded boundary conditions.

4.16.4 End Conditions

For a fixed-Free column the critical load becomes:

Pcr=π 2*E*A4*Sr2 and the solution becomes y=C1*sin(π*x2*L)

→ note now that the critical load is now ¼ of the pinned-pinned column.

Slenderness  ratio can be accounted for the different boundary conditions using the effective length (leff).

→ Please see the table in you book that describes the different effective lengths for the different boundary conditions.

4.16.5 Intermediate Columns

Sometimes the material’s compressive yield strength, Syc, is used as the value of σxin the axial stress equation and the for critical unit load equation.

→ If this is the case, sometimes the part will still fail inside the empirical envelope. To compensate this we use:

Pcr=Syc-1E*Syc*Sr2*π2*A

4.16.5 Eccentric Columns

→ If you’re interested, please read on your own accord.

4.17 Stresses In Cylinders

-Cylinders are often used as pressure vessels or pipelines and can be subjected to internal and/or external pressures.

-Some of these devices can be open-ended and some are closed-ended.

-If open-ended, a two-dimensional stress state will exist in the cylinder walls, with radial and tangential (hoop) stress components.

-An example of an open-ended cylinder would be the barrel of a potato gun, some fluid storage tanks, nozzle for the water jet in fabrication shop, etc…

-If closed-ended, a third-dimensional stress called longitudinal or axial will also be present. These three applied stresses are mutually orthogonal and are principal, since there is no applied shear from the uniformly distributed pressure.

-An air compressor, Vacuum tube for a laser cutter, submarine, etc…

4.17.1 Thick- Wall Thickness

Open-Ended Cylinder

pi=internal pressure

po=external pressure

ri=inner radius

ro=outer radius

r=radius of interest

σtangent=σt=pi*ri2-po*ro2ro2-ri2+ri2*ro2*(pi-po)r2*(ro2-ri2)

σradial=σr=pi*ri2-po*ro2ro2-ri2-ri2*ro2*(pi-po)r2*(ro2-ri2)

If the ends are closed:

σaxial=σa=pi*ri2-po*ro2ro2-ri2

If external pressure is 0:

σt=ri2*piro2-ri2*(1+ro2ri2)

σr=ri2*piro2-ri2*(1-ro2ri2)

If external pressure is 0 and closed ended:

σa=ri2*piro2-ri2

4.17.2 Thin-Walled Cylinders

*This only applies when the wall thickness is less than about 1/10 of the radius.

Open-Ended Cylinder

σt=P*rt

σr=0

Closed-Ended Cylinder

σa=P*r2*t

Pressure vessels can be extremely dangerous even at relatively low pressures if the stored volume is large and the pressurized medium is compressible.

Large amounts of energy can be released suddenly at failure, possibly causing serious injury.