Chapter 12 Spur Gears

12.0 Introduction

Gears are used to transmit torque and angular velocity in a wide variety of applications.

This chapter will deal with the simplest type of gear, the spur gear, designed to operate on parallel shafts and having teeth parallel to the shaft axis.

Other gear types such as helical, bevel, and worm can accommodate non-parallel shafts.

The American Gear Manufacturers Association (AGMA) supports research in gear design, materials, and manufacturing and publishes standards for their design, manufacture, and assembly.

→ Gears are now highly standardized as to tooth shape and size.

Early gears were most likely made crudely of wood and other easily worked materials, their teeth merely being pegs inserted in a disk or wheel.

It was not until the industrial revolution that machines demanded, and manufacturing techniques allowed, the creation of gears as we now know them with specially shaped teeth formed or cut into a metal disk.

12.1 Gear Tooth Theory

The simplest means of transferring rotary motion from one shaft to another is a pair of rolling cylinders.

They may be an external set of rolling cylinders or an internal set.

There will be no slip between the cylinders until the maximum available frictional force at the joint is exceeded by the demands of torque transfer.

The principal drawbacks to the rolling-cylinder drive mechanism are its relatively low torque capability and the possibility of slip.

On the flip side, the gear train (meshed teeth of the rolling cylinders) allows absolute phasing of the input and output shafts for timing purposes.

→ Meshed gears are called gearset.

When two gears are placed in mesh to form a gearset, it is conventional to refer to the smaller of the two gears as the pinion and the other one is called gear.

12.1.1 The Fundamental Law of Gearing

Fundamental Law of Gearing

→ the angular velocity ratio between the gears of a gearset must remain constant throughout the mesh.

Angular velocity ratio mV  is equal to the ratio of the pitch radius of the input gear to that of the output gear.

mV=ωoutωin= ±rinrout= ±dindout						(12.1a)

The pitch radii in equation 12.1a are those of the rolling cylinders to which we are adding the teeth.

The positive or negative sign accounts for internal or external cylinder sets as show in the figure above.

An external set reverses the direction of rotation between the cylinders and requires the negative sign. An internal gearset (and a belt or chain drive) will have the same direction of rotation on input and output shafts and require the positive sign in equation 12.1a.

The surfaces of the rolling cylinders will become the pitch circles, and their diameters the pitch diameters of the gears.

The contact point between the cylinders lies on the line of centers and this point is called the pitch point.

Torque ratio and mechanical advantage mA is the reciprocal of the velocity ratio mV:

mA=1mV=ωinωout= ±routrin= ±doutdin					(12.1b)

Thus a gearset is essentially a device to exchange torque for velocity or vice versa.

it is usually desirable to maintain a constant ratio between the gears as they rotate. Any variation in ratio will show up as oscillation in the output velocity and torque, even if the input is constant with time.

Gear ratio, mG, is taken as the magnitude of either the velocity ratio or the torque ratio, whichever is >1.

mG=|mV| or mG=|mA|, for mG≥1				(12.1c)

→ The gear ratio is always a positive number > 1 regardless of the direction in which the power flows through the gearset.

In order for the fundamental law of gearing to be true, the gear tooth contours on mating teeth must be conjugates of one another.

→ most gears use the involute of a circle for their shape.

11.1.2 The Involute Tooth Form

The involute of a circle is a curve that can be generated by unwrapping a taut string from a cylinder:

1. The string is always tangent to the base circle.

2. The center of curvature of the involute is always at the point of tangency of the string with the base circle.

3. A tangent to the involute is always normal to the string, which is the instantaneous radius of curvature of the involute curve.

The figure with the two meshing gears shows two involutes on separate cylinders in contact or “in mesh.”

→ The cylinders from which the strings are unwrapped are called the base circles of the respective gears.

→ Note that the base circles are necessarily smaller than the pitch circles, which are at the radii of the original rolling cylinders, rp and rg.

→ the involute only exists outside of the base circle.

→ The amount of tooth that sticks out above the pitch circle is the addendum, shown as ap and ag for pinion and gear, respectively.

The line of action, always passes through the pitch point regardless of where in the mesh the two teeth are contacting.

→ The pitch point has the same linear velocity in both pinion and gear, called the pitch-line velocity.

→ The angle between the line of action and the velocity vector is the pressure angle φ.

11.1.3 Pressure Angle

The pressure angle φ in a gearset is defined as the angle between the line of action (common normal) and the direction of velocity at the pitch point such that the line of action is rotated φ degrees in the direction of rotation of the driven gear.

→ Pressure angles of gearsets are standardized at a few values by the gear manufacturers.

→ The standard values are 14.5, 20 and 25o, with 20o being the most commonly used and 14.5o now being obsolete.

11.1.4 Gear Mesh Geometry

Fundamental Law of Gearing

→ the common normal of the tooth profiles, at all contact points within the mesh, must always pass through a fixed point on the line of centers, called the pitch point.

→ It is this property of the involute that causes it to obey the fundamental law of gearing.

The distance along the line of action between these points, beginning and leaving contact when the pinion and gear are meshed, within the mesh is called the length of action Z, defined by the intersections of the respective addendum circles with the line of action.

→ The distance along the pitch circle within the mesh is the arc of action, and the angles subtended by these points and the line of centers are the angle of approach and angle of recess.

→ The arc of action on both pinion and gear pitch circles must be the same length for zero slip between the theoretical rolling cylinders.

Z=rp+ap2-rp*cosφ2+rg+ag2-(rg*cosφ)2-C*sinφ		(12.2)

rp and rg  are the pitch circle radii

ap and ag are the addenda of pinion and gear, respectively

C is the center distance between pinion and gear

φ is the pressure angle

12.1.5 Rack and Pinion

If the diameter of the base circle of a gear is increased without limit, the base circle will become a straight line. If the “string” wrapped around this base circle to generate the involute were still in place after the base circle’s enlargement to an infinite radius, the string would be pivoted at infinity and would generate an involute that is a straight line.

→ This linear gear is called a rack.

→ The teeth of the rack gear are trapezoids.

→ Rack Gears are used to translate a rotary motion to a linear motion.

→ Like in older car steering systems, which used and multi-bar linkage system.

→ Not only are rack gears used to translating a rotary motion to a linear one, but they can also be cutter to make radial involute gears.

12.1.6 Changing Center Distance

The pitch circle for a gear is the theoretical circle in which two gears mate together to form a gearset.There will be an ideal center distance that will give us the nominal pitch diameters for which the gears were designed.

However, the limitations of the manufacturing process give a low probability that we will be able to exactly achieve this ideal center distance in every case. Most likely, there will be some error in the center distance, even if small.

With an involute tooth form, center-distance errors do not affect the velocity ratio.

→ This is the principal advantage of the involute over all other possible tooth forms and is the reason why it is nearly universally used for gear teeth.

→ Only the pressure angle is affected by the change in center distance.

→ The pitch point has moved in proportion to the shifts of the center distance and the pitch radii.

→ ...the velocity ratio of involute gears is fixed by the ratio of their base-circle diameters, which are unchanging once the gear is cut.?

→ If the gear tooth form is not and involute, then an error in center distance will cause variation, or “ripple,” in the output velocity. The output angular velocity will then not be constant for a constant input velocity, violating the fundamental law of gearing.

12.1.7 Backlash

Another factor affected by changing center distance C is backlash.

Backlash is defined as the gap between mating teeth measured along the circumference of the pitch circle.

→ gap between teeth can cause impact loading on the teeth, which reduces longevity of the teeth. As well as increasing stresses and wear, backlash can cause undesirable positional error in some applications.

→ As long as the gearset is run with a non-reversing torque, then backlash should not be a problem.

12.1.8 Relative Tooth Motion

The relative motion between involute teeth is pure rolling at the pitch point.

At points on the tooth away from the pitch point, some sliding occurs in combination with rolling.

→ The average amount of sliding in an involute-tooth mesh is about 9%, as discussed in Section 7.13.

→ The surface stresses are increased by the sliding component, as discussed in Section 7.11. Table 7-7 shows surface fatigue-strength data developed from extensive testing of rolling plus 9% sliding between various material combinations.

12.2 Gear Tooth Nomenclature

The tooth height is defined by the addendum (added on) and the dedendum (subtracted from), which are referenced to the nominal pitch circle.

→ The dedendum is slightly larger than the addendum to provide a small amount of clearance between the tip of one mating tooth (addendum circle) and the bottom of the tooth space of the other (dedendum circle).

→ The working depth of the tooth is twice the addendum and the whole depth is the sum of addendum and dedendum.

The tooth thickness is measured at the pitch circle, and the tooth space width is slightly larger than the tooth thickness.

→ The difference between these two dimensions is the backlash.

The circular pitch, Pc , is the arc length along the pitch circle circumference measured from a point on one tooth to the same point on the next.

Pc=π*dN								(12.3a)

d= Pitch diameter

N= number of teeth

The diametral pitch, Pd ,number of teeth. A more convenient way to define tooth size is to relate it directly to the diameter d of the pitch circle rather than to its circumference.

Pd=Nd=# of teethinch-diameter=πPc						(12.4a &4b)

The base pitch, Pb , is the tooth pitch can also be measured along the base circle circumference.

Pb=Pc*cosφ=πPd*cosφ						(12.3b)

The SI system, for metric gears, defines a parameter called module, which is the reciprocal of diametral pitch

m=dN=milimeter-diameter# of teeth					(12.4c)

→ metric gears are not interchangeable with U.S. gears

Conversion → m=25.4Pd					(12.4d)

The velocity ratio, mV :

mV=ωoutωin= ±rinrout= ±dindout= ±NinNout					(12.5a)

Mechanical Advantage, mG , which is the reciprocal of the velocity ratio:

mG=NgNp									(12.5b)

Figure Above - Gear Anatomy

Table 12-1 AGMA Full-Depth Gear Tooth Specifications

| Parameter | Coarse Pitch (Pd < 20) | Fine Pitch (Pd ≥ 20) |
| --- | --- | --- |
| Pressure Angle φ | 20o or 25o | 20o |
| Addendum a | 1.000Pd | 1.000Pd |
| Dedendum d | 1.250Pd | 1.250Pd |
| Working Depth | 2.000Pd | 2.000Pd |
| Whole Depth | 2.250Pd | 2.200Pd+0.002 in |
| Circular Tooth Thickness | 1.571Pd | 1.571Pd |
| Fillet Radius- Basic Rack | 0.300Pd | Not Standardized |
| Minimum Basic Clearance | 0.250Pd | 2.200Pd+0.002 in |
| Minimum width of Top land | 0.250Pd | Not Standardizes |
| Clearance (shaved or ground teeth) | 0.350Pd | 0.350Pd+0.002 in |
| Base Pitch Circle | πPd*cosφ | πPd*cosφ |
| Base Circle | π*d*cosφ | π*d*cosφ |

Table 12-2 Standard Diametral Pitches

| Coarse (Pd < 20) | Fine (Pd ≥20) |
| --- | --- |
| 1 | 20 |
| 1.25 | 24 |
| 1.5 | 32 |
| 1.75 | 48 |
| 2 | 64 |
| 2.5 | 72 |
| 3 | 80 |
| 4 | 96 |
| 5 | 120 |
| 6 |  |
| 8 |  |
| 10 |  |
| 12 |  |
| 14 |  |
| 16 |  |
| 18 |  |

Conversion from inches to metric:

m=25.4Pd									(12.4d)

Table 12-3 Standard Metric Modules

| Metric Module (mm) | Equivalent Pd (in-1) |
| --- | --- |
| 0.3 | 84.67 |
| 0.4 | 63.50 |
| 0.5 | 50.80 |
| 0.8 | 31.75 |
| 1 | 25.4 |
| 1.25 | 20.32 |
| 1.5 | 16.93 |
| 2 | 12.70 |
| 3 | 8.47 |
| 4 | 6.35 |
| 5 | 5.08 |
| 6 | 4.23 |
| 8 | 3.18 |
| 10 | 2.54 |
| 12 | 2.12 |
| 16 | 1.59 |
| 20 | 1.27 |
| 25 | 1.02 |

12.3 Interference and Undercutting

Undercutting weakens the tooth by removing material at its root.

→ Severe undercutting will cause early tooth failure.

Interference happens when there are too few teeth.

The minimum number of full-depth teeth required to avoid interference on a pinion running against a standard rack can be calculated from:

Nmin=2sin 2φ								(12.6)

Table 12-4 Minimum Number of Pinion Teeth to avoid Interference between full depth pinion and full depth rack gear

| Pressure Angle (deg) | Nminimum |
| --- | --- |
| 14.5 | 32 |
| 20 | 18 |
| 25 | 12 |

Table 12-5 Minimum Number of Pinion teeth to avoid interference between a 20o Full depth pinion and Full-Depth Gears of Various Sizes

| Minimum # of Pinion Teeth | Maximum # of Gear Teeth |
| --- | --- |
| 17 | 1,306 |
| 16 | 101 |
| 15 | 45 |
| 14 | 26 |
| 13 | 16 |

12.3.1 Unequal-Addendum Tooth Forms

If the addendum is longer on the pinion and if the addendum is shorter on the gear, this can also cause an interference too.

12.4 Contact Ratio

The contact ratio mP , defines the average number of teeth in contact at any one time.

mP=ZPb=Pd*Zπ*cosφ							(12.7a &b)

If the contact ratio is 1, then one tooth is leaving contact just as the next is beginning contact.

→ For contact ratios between 1 and 2, which are common for spur gears, there will still be times during the mesh when one pair of teeth will be taking the entire load.

→ The minimum acceptable contact ratio for smooth operation is 1.2.

→ A minimum contact ratio of 1.4 is preferred, and larger is better.

→ Most spur gear-sets will have contact ratios between 1.4 and 2.

12.5 Gear Trains

A gear train is any collection of two or more meshing gears.

Gear trains may be simple, compound, or epicyclic.

A pair of gears, or gear-set, is then the simplest form of gear train and is usually limited to a ratio of about 10:1.

The gear-set will become large and hard to package beyond that ratio if the pinion is kept above the minimum numbers of teeth.

12.5.1 Simple gear Trains

A simple gear train is one in which each shaft carries only one gear, the most basic two gear example of which is shown in the figure below.

These gear trains are 1-DOF devices, one input and one output.

The velocity ratio of a gear-set is given by equation 12.5a, shows a simple gear train with five gears in series.

mV=-N2N3*-N3N4*-N4N5*-N5N6= +N2N6		(12.8)

→ the numerical effects of all gears (in a simple gear-set) except the first and last cancel out.

→ Only the sign of the overall train ratio is affected by the intermediate gears, which are called idlers, because no power is typically taken from their shafts.

→ if the sign is positive, the input and output gear are moving in the same direction.

→ if the sign is negative, the input and output gear are moving in the opposite direction.

12.5.2 Compound Gear Trains

A compound train is one in which at least one shaft carries more than one gear.

→ They’re used for gear train ratios that are greater than 10:1.

→ Unless they use a gear train called epicycle.

mV=-N2N3*-N4N5					(12.9a)

The gear train on the left is a Non-reverted gear train and the gear train on the right is a reverted gear train.

The general expression for gear train ratio:

mV= ±product of number of teeth on driver gearsproduct of number of teeth on driven gears				(12.9b)

→ A larger ratio can be obtained in a compound gear train despite the approximately 10:1 limitation on individual gear-set ratios.

→ The plus or minus sign in equation 12.9b depends on the number and type of meshes in the train, whether external or internal.

12.5.3 Reverted Compound Trains

Reverted Compound Train is when the output shaft is concentric with the input shaft.

→ “bringing it back on to itself”

Non-Reverted Compound Train is when the output shaft is not concentric or coincident with the input shaft.

12.5.4 Epicycle or Planetary Gear Trains

Epicycle or planetary train are 2-DOF devices in which they either need two inputs for one output or one input and two outputs.

→ automotive differential has one input is provided (the driveshaft) and two frictionally coupled outputs are obtained (two driving wheels).

Epicycle or planetary trains have several advantages over conventional trains, among which are higher train ratios obtainable in smaller packages, reversion by default, and simultaneous, concentric, bidirectional outputs available from a single unidirectional input.

This has become an epicycle train with a sun gear and a planet gear orbiting around the sun, held in orbit by the arm.

A conventional gear train is a special case of the epicyclic gear train by holding the arm stationary.

A more useful configuration is shown below, to which a ring gear has been added. The ring gear meshes with the planet and pivots concentric with the pinion, so it can be easily tapped as the output member.

→ Most planetary trains will be arranged with ring gears to bring the planetary motion back to a grounded pivot.

→ Note how the sun gear, ring gear, and arm are all brought out as concentric hollow shafts so that each can be accessed to tap its angular velocity and torque as either an input or an output.

Since the gears are rotating with respect to the arm, and the arm itself has motion, the velocity-difference equation must be used:

ωgear=ωarm+ωgear/arm						(12.10)

Equations 12.10 and 12.5a are all that are needed to solve for the velocities in an epicyclic train, provided that the tooth numbers and two input conditions are known.

For the first gear in the system:

ωF/arm=ωF-ωarm							(12.11a)

For the last gear in the system

ωL/arm=ωL-ωarm							(12.11b)

Dividing the last by the first:

ωL/armωF/arm=ωL-ωarmωF-ωarm= ±product of number of teeth on driver gearsproduct of number of teeth on driven gears=mV		(12.11c &.12)

12.6 Gear Manufacturing

Two Categories

→ Machining(roughing and machining) are material removal techniques.

→ Roughing: milling, rack cutter, a shaper cutter, or a hob.

→ Finishing: shaving, burnishing, lapping, honing, or grinding.

→ Forming refers to the direct casting, molding, drawing, or extrusion of tooth forms in molten, powdered, or heat-softened materials.

12.6.1 Forming Gear Teeth

The teeth on the gear are formed all at once from a mold or die into which the tooth shapes have been machined.

→ The accuracy of the teeth is entirely dependent on the quality of the die or mold and in general is much less than can be obtained from roughing or finishing methods.

→ Most of these methods have high tooling costs, making them suitable only for high production quantities.

12.6.1.1 Casting

Teeth can be sand cast or die cast in various metals. The advantage is low cost, as the tooth shape is built into the mold.

→ The resulting teeth are of low precision and are used only in non-critical applications such as toys and small appliances or cement-mixer barrels, where noise and excessive backlash are not detrimental to operation.

→ Die casting provides a better surface finish and accuracy than sand casting but has high tooling costs, requiring large production volume to justify its use.

12.6.1.2 Investment Casting (lost-wax cast)

The mold is made of a refractory material that allows high melt-temperature materials to be cast.

12.6.1.3 Sintering

Powdered metals (PM) are pressed into a gear-shaped metal mold cavity, removed, and heat-treated (sintered) to increase their strength.

→ This technique is typically used for small-sized gears.

12.6.1.4 Injection Molding

Used to make non-metallic gears in various thermoplastics such as nylon and acetal.

→ These are low-precision gears in small sizes but they have the advantages of low cost and the ability to be run without lubricant at light loads.

12.6.1.5 Extruding

Used to form teeth on long rods, which are then cut into usable lengths and machined for bores and keyways, etc.

12.6.1.6 Cold Drawing

Forms teeth on steel rods by drawing them through hardened dies.

→ The cold working increases strength and reduces ductility.

12.6.1.7 Stamping

Sheet metal can be stamped with tooth shapes to form low-precision gears at low cost in high quantities.

12.6.2 Machining

→ Roughing processes include milling the tooth shape, with formed cutters or generating the shape with a rack cutter, a sharper cutter, or a hob.

→ Finishing processes include shaving, burnishing, lapping, honing, or grinding.

12.6.3 Roughing Processes

12.6.3.1 Form Milling

The rotating cutter is plunged into the blank to cut one tooth at a time.

→ The gear blank is then rotated through one circular pitch and the next tooth cut.

12.6.3.2 Rack Generation

A rack cutter for any involute pitch can be easily made, since its tooth shape is a trapezoid.

→ The hardened and sharpened rack is then reciprocated along the axis of the gear blank and fed into it while being rotated around the gear blank so as to generate the involute tooth on the gear.

12.6.3.3 Gear Shaping

Uses a cutting tool in the shape of a gear, which is reciprocated axially across the gear blank to cut the teeth while the blank rotates around the shaper tool.

12.6.3.4 Hobbing

Is analogous to a thread tap. Its teeth are shaped to match the tooth space and are interrupted with grooves to provide cutting surfaces.

12.6.4 Finishing Processes

Finishing operations typically remove little or no material but improve dimensional accuracy, surface finish, and/or hardness.

12.6.4.1 Shaving

Is similar to gear shaping, but uses accurate shaving tools to remove small amounts of material from a roughed gear to correct profile errors and improve finish.

12.6.4.2 Grinding

Uses a contoured grinding wheel that is passed over the machined surface of the gear teeth, typically computer-controlled, to remove small amounts of material and improve surface finish.

12.6.4.3 Burnishing

Runs the rough-machined gear against a specially hardened gear.

12.6.4.4 Lapping and Honing

Both employ an abrasive-impregnated gear or gear-shaped tool that is run against the gear to abrade the surface.

12.6.5 Gear Quality

The AGMA Standard 2000-A88 defines dimensional tolerances for gear teeth and a quality index QV that ranges from the lowest quality (3) to the highest precision (16).

→ Formed gears 3-4

→ Roughing gears 5-7

→ shaving or grinding 8-11

→ Lapping and honing 11+

Gear quality can have a significant effect on the load sharing between teeth.

→ If the tooth spacings are not accurate and uniform, the teeth in the mesh will not all be in simultaneous contact.

12.7 Loading on Spur Gears

The force- loading(W) on a gear is in two components:

Tangential component	Wt=TPrP=2*TPdp=2*pd*TpNP

TP= Torque on the pinion shaft

rP= pitch radius on pinion

dP= pitch diameter on the pinion

NP= Number of teeth on pinion

pd= diametral pitch of the pinion

φ = pressure angle

Radial Component		Wr=Wt*tan φ

Resultant Force W, 		W=Wtcos φ

12.8 Stresses in Spur Gears

There are two modes of failure that affect gear teeth: fatigue fracture due to fluctuating bending stresses at the root of the tooth and surface fatigue (pitting) of the tooth surfaces.

12.8.1 Bending Stresses

The Lewis Equation, recognized that the tooth is a cantilever beam with its critical section at the root. No longer used in its original form, but it serves as the basis for a more modern version.

σb=Wt*pdF*Y

Y is a dimensionless geometry factor, Lewis form factor

Still valid, but it has been augmented with additional factors to account for failure mechanisms only later understood.

Bending Stress Equation is only valid for certain assumptions about the tooth and gear-mesh geometry:

- The contact ratio is between 1 and 2

  - Assumption 1 is then conservative with larger contact ratios

- There is no interference between the tips and root fillets of mating teeth and no undercutting of teeth above the theoretical start of the active profile.

- No teeth are pointed

- There is nonzero backlash

  - Assumption 4 recognizes that gears with zero backlash will not run freely together due to excess friction.

- The root fillets are standard, assumed smooth, and produced by a generating process.

  - Assumption 5 accounts for the use of stress-concentration factors for root fillets based on work by Dolan and Broghammer.

- The friction forces are neglected.

Bending Stress equation for US and SI specification gears

US	σb=Wt*pdF*JKa*KmKV*Ks*KB*KI

J is substituted for his form factor Y, Tables 12-8 through 12-15

U on tables is undercutting

SI 	σb=WtF*m*JKa*KmKV*Ks*KB*KI

m is the metric module.

K factors are modifiers to account for various conditions.

Bending Strength Geometry Factor J

The geometry factor J can be calculated from a complicated algorithm defined in AGMA Standard 908-B89. The same standard also provides tables of J factors for standard, full-depth teeth and for 25% and 50% unequal-addendum teeth, all with 14.5, 20, and 25o pressure angles.

J factors are different for the pinion and gear in each mesh combination.

Dynamic Factor KV

Dynamic factor KV attempts to account for internally generated vibration loads from tooth-tooth impacts induced by nonconjugate meshing of the gear teeth.

-These vibration loads are called transmission error and will be worse with low-accuracy gears.

-Precision gears will more closely approach the ideal of smooth, constant-velocity-ratio torque transmission.

US	KV=AA+VtB

SI	KV=AA+ 200*VtB

Vt = pitch-line velocity of the gear mesh in units of ft/min or m/s

US	(Vt )max= [A+(QV - 3)] 2 ft/min

SI	(Vt)max=A + (QV-3) 2200   m/s

A=50+56*(1-B)

B=(12-QV)2/34  for 6≤QV≤11

QV is the quality index of the lower-quality gear in the mesh.

For gears with QV≤5,

US 	KV=5050 + Vt

SI	KV=5050 + 200*Vt

Vt =< 2500 ft/min (13 m/s)

Load Distribution Factor Km

Any axial misalignment or axial deviation in tooth form will cause the transmitted load Wt to be unevenly distributed over the face width of the gear teeth. This problem becomes more pronounced with larger face widths.

-An approximate and conservative way to account for less than uniform load distribution is to apply the factor Km to increase the stresses for larger face widths.

-A useful rule of thumb is to keep the face width F of a spur gear within the limits 8/pd < F < 16/pd , with a nominal value of 12/ pd.

Application Factor Ka

The fluctuating moments on the teeth described in section 12.7 are due to the teeth coming into and out of mesh under a uniform or average load.

If either the driving or driven machine has time-varying torques or forces, then these will increase the loading felt by the gear teeth above the average values.

Ka can be applied to increase the tooth stress based on the “shockiness” of the machinery connected to the gear train.

Size Factors KS

If the part being designed is larger than 0.3 inches in diameter, it may be weaker than indicated by the test data. The KS can be set to 1 unless the designer wishes to raise its values to account for particular situations, such as very large teeth.

A value of 1.25 to 1.5 would be conservative assumption in such cases.

Rim Thickness Factor KB

This factor is to account for situations in which a large-diameter gear, made with rim and spokes rather than as a solid disk.

rim-tooth ratio mB 	mB=tR/ ht

KB= -2*mB+3.4     0.5 ≤mB≤1.2

KB = 1.0 		mB > 1.2

Idler Factor KI

An idler gear is subjected both to more cycles of stress per unit time and to larger-magnitude alternating loads than its non-idler cousins.

To account for this situation, the factor KI is set to 1.42 for an idler gear or 1.0 for a non-idler gear.

12.7.1 Surface Stresses

Mating gear teeth have a combination of rolling and sliding at their interference. At the pitch point, their relative motion is pure rolling. The percentage of sliding increases with distance from the pitch point.

The stresses at the tooth surface are dynamic Hertzian contact stresses in combined rolling and sliding. These stresses are three-dimensional and have peak values either at the surface or slightly below it, depending on the amount of sliding present in combination with rolling.

Surface Stresses	σc=CP* WtF*I*dCa*CmCV*Cs*Cf

Wt = tangential force on the tooth

d = is the pitch diameter of the smallest of the two gears in mesh

F = is the face width

I = dimensionless surface geometry factor for pitting resistance

CP = elastic coefficient

Ca = Ka application factor

Cm = Km Load distribution factor

CV = KV Dynamic Factor

CS = KS Size Factor

Surface Geometry Factor I

This factor takes into account the radii of curvature of the gear teeth and the pressure angle

I=cos φ1ρp ± 1ρg * dp

ρp= radii of curvature of the pinion

ρg= radii of curvature of the gear

ρp= rp + 1+xppd2-(rp*cosφ)2 - πPd*cos φ

xP= pinion addendum coefficient, equal to percentage. For full-depth teeth xP = 0.

For 25% long addendum teeth xp=0.25

ρg= C*sinφ ∓ρp

for the minus/ plus sign, minus for external, plus for internal.

Elastic Coefficient CP

Accounts for the difference in tooth materials and is found from:

CP=1π*1-vp2Ep +  1 - vg2Eg	units of (psi)0.5  or (MPa)0.5

EP and Eg are the respective modulus of elasticity for pinion and gear

vp and vg are their respective Poisson’s ratios.

Surface Finish Factor CF

Used to account for unusually rough surface finishes on gear teeth.

AGMA has not yet established standards for surface-finish factors and recommends that Cf be set to 1 for gears made by conventional methods.

12.9 Gear Materials

Steels, Cast Irons, and malleable and nodular irons are the most common choices for gears.

Some gears need surface hardening through nitriding, which will strengthen the material a well as improve wear resistance.

Some gears are made from injection-molded thermoplastics such as nylon, acetal, ABS, and Delrin. Non-metallic gears have very low noise but are limited in torque capacity by their low material strengths.

12.9.1 Material Strengths

Use the methods for estimating fatigue strength outlined in chapter 6 (fatigue loading).

12.9.2 Bending - Fatigue Strengths for Gear Materials

The AGMA bending-fatigue strength data are all stated at 1E7 cycles of repeated stress (rather than the 1E6 or 5E8 cycles sometimes used for other materials), and for a 99% reliability level ( rather than the 50% reliability common for general fatigue and static strength data). These strengths are compared to the peak stress σb calculated from the US and SI bending stress equations using the load Wt.

The correction formula for the bending-fatigue strength for gears is

Sfb= KLKT*KR*Sfb'

Sfb’ is the uncorrected strength

KL Life Factor

KT Temperature Factor

KR Reliability Factor

Life Factor KL is dependent on the number of load cycles, number of mesh contacts, under load, and gear tooth being analyzed.

Temperature Factor KT

For steel gears using lubrication:

KT = 1 for T< 250oF

KT = (460 +TF) / 620 for T> 250oF

TF is the temperature of the lubrication oil

Reliability Factor KR

12.9.3 Surface- Fatigue Strengths for Gear Materials

Sfc=CL*CHCT*CR*Sfc'

CT = KT , Temperature correction factor

CR = KR , Reliability Correction Factor

CL = KL Life Factor

CH = hardness - ratio factor for pitting resistance.

CH = 1 + A*(mG - 1)	For Through hardened pinions - through gears

if HBP / HBg < 1.2 then A=0

if 1.2 =< (HBP / HBg ) =< 1.7 then A = 0.00898* (HBP / HBg) -0.00829

if (HBP / HBg)> 1.7 then A = 0.00698

CH = 1 + B*(450 - HBg)

US	B = 0.00075*e -0.0112*Rq

SI	B = 0.00075*e -0.052*Rq

Rq is the rms surface roughness of the pinion μin

12.10 Lubrication of Gearing

*Up to students to read

12.11 Design of Spur Gears

*Up to students to read

12.12 Case Study

* Up to students to read